School of Engineering & Technology

T.Y. B. Tech. CSE(AI&DS)

Course: Data Communication And Computer Networking

Report On:

Campus Network Design And Implementation Using EIGRP And DHCP

Submitted by:

Sakshi  Bodke

Sakshi Chidre

Tejas Shinde

Yash Shinde

Under the Guidance of:

Dr. Manisha Khadse

Year: 2025- 2026

CAMPUS NETWORK DESIGN AND IMPLEMENTATION USING EIGRP AND DHCP

Problem Statement:

Problem Statement

Educational institutions face significant challenges in establishing efficient network infrastructure that can support multiple departments, buildings, and services. Traditional static IP configuration becomes unmanageable in large-scale deployments, leading to configuration errors, IP conflicts, and administrative overhead. Additionally, without dynamic routing protocols, network administrators must manually configure routes, making the network inflexible and difficult to scale.

Objectives:

- To design an efficient and scalable campus network connecting multiple departments.

- To implement centralized IP address management using a single DHCP server.

- To configure EIGRP (Enhanced Interior Gateway Routing Protocol) for dynamic routing between routers.

- To enable seamless inter-department communication and data sharing.

- To provide essential network services such as DNS, Web, and FTP servers.

- To ensure reliability, reduced network traffic, and fast convergence in the campus network.

Scope:

- The project covers the design and implementation of a LAN-based campus network with inter-building connectivity.

- It involves setting up three routers, multiple departmental switches, and end devices (PCs, printers, laptops).

- The implementation ensures IP allocation using DHCP, name resolution using DNS, and data communication via EIGRP routing.

- The network simulates real-world institutional setups including buildings like A,B, Internet Lab, Admin, and PVC Cabin.

- The project aims to serve as a model for future scalability and real deployment.

Design & Topology Diagram:

| DEPARTMENT | COMPONENT | IP ADDRESS | DEFAULT GATEWAY | DNS SERVER | PRINTER QUANTITY |
| --- | --- | --- | --- | --- | --- |
| SERVER ROOM (192.168.1.0) | DHCP SERVER | 192.168.1.2 | 192.168.1.1 | 192.168.1.3 | - |
|  | DNS SERVER | 192.168.1.3 |  |  |  |
|  | WEB SERVER | 192.168.1.4 |  |  |  |
|  | FTP SERVER | 192.168.1.5 |  |  |  |
|  | PC1 | 192.168.1.10 |  |  |  |

| DEPARTMENT | COMPONENT | IP ADDRESS | DEFAULT GATEWAY | DNS SERVER | Printer QUANTITY |
| --- | --- | --- | --- | --- | --- |
| Building A (192.168.2.0) | A- HOD CABIN | 192.168.2.11 | 192.168.2.1 | 192.168.1.3 | 1 |
|  | A- LAB 1 | 192.168.2.12 |  |  |  |
|  | A- LAB 2 | 192.168.2.13 |  |  |  |
|  | A- LAB 3 | 192.168.2.14 |  |  |  |
|  | A- LAB 4 | 192.168.2.15 |  |  |  |

| DEPARTMENT | COMPONENT | IP ADDRESS | DEFAULT GATEWAY | DNS SERVER | PRINTER QUANTITY |
| --- | --- | --- | --- | --- | --- |
| Building B (192.168.3.0) | B- HOD CABIN | 192.168.3.11 | 192.168.3.1 | 192.168.1.3 | 1 |
|  | B- LAB 1 | 192.168.3.12 |  |  |  |
|  | B- LAB 2 | 192.168.3.13 |  |  |  |
|  | B- LAB 3 | 192.168.3.14 |  |  |  |
|  | B- LAB 4 | 192.168.3.15 |  |  |  |

| DEPARTMENT | COMPONENT | IP ADDRESS | DEFAULT GATEWAY | DNS SERVER | PRINTER QUANTITY |
| --- | --- | --- | --- | --- | --- |
| Internet Lab (192.168.4.0) | PC2 | 192.168.4.10 | 192.168.4.1 | 192.168.1.3 | 1 |
|  | PC3 | 192.168.4.11 |  |  |  |
|  | PC4 | 192.168.4.12 |  |  |  |
|  | PC5 | 192.168.4.13 |  |  |  |

| DEPARTMENT | COMPONENT | IP ADDRESS | DEFAULT GATEWAY | DNS SERVER | PRINTER QUANTITY |
| --- | --- | --- | --- | --- | --- |
| PVC Cabin (192.168.6.0) | PC 0 | 192.168.6.11 | 192.168.6.1 | 192.168.1.3 | x |
|  | LAPTOP 0 | 192.168.6.12 |  |  |  |

| DEPARTMENT | COMPONENT | IP ADDRESS | DEFAULT GATEWAY | DNS SERVER | PRINTER QUANTITY |
| --- | --- | --- | --- | --- | --- |
| Admin Building (192.168.5.0) | OFFICE | 192.168.5.13 | 192.168.5.1 | 192.168.1.3 | 3 |
|  | EXAM CELL | 192.168.5.14 |  |  |  |
|  | ENQUIRY | 192.168.5.15 |  |  |  |
|  | TnP | 192.168.5.16 |  |  |  |

Configuration Details:

Router1:

Router>enable

Router#

Router#configure terminal

Router(config)#interface Serial0/1/0

Router(config-if)#exit

Router(config)#router eigrp 1

Router(config-router)#network 192.168.1.0 255.255.255.0

Router(config-router)#network 192.168.4.0 255.255.255.0

Router(config-router)#network 201.151.12.0 255.255.255.0

Router(config-if)#exit

Router(config)#interface GigabitEthernet0/1

Router(config-if)#ip helper-address 192.168.1.2

Router2:

Router>enable

Router#

Router#configure terminal

Router(config)#interface Serial0/1/0

Router(config)#router eigrp 1

Router(config-router)#network 192.168.5.0 255.255.255.0

Router(config-router)#network 192.168.6.0 255.255.255.0

Router(config-router)#network 201.151.12.0 255.255.255.0

Router(config-router)#network 201.151.11.0 255.255.255.0

Router#

Router#configure terminal

Router(config)#interface GigabitEthernet0/0

Router(config-if)#ip helper-address 192.168.1.2

Router(config)#interface GigabitEthernet0/1

Router(config-if)#ip helper-address 192.168.1.2

Router3:

Router(config-if)#exit

Router(config)#interface Serial0/1/0

Router(config-if)#exit

Router(config)#router eigrp 1

Router(config-router)#network 192.168.3.0 255.255.255.0

Router(config-router)#network 192.168.2.0 255.255.255.0

Router(config-router)#network 201.151.11.0 255.255.255.0

Router>enable

Router#

Router#configure terminal

Router(config)#interface GigabitEthernet0/0

Router(config-if)#ip helper-address 192.168.1.2

Router(config)#interface GigabitEthernet0/1 Router(config-if)#ip helper-address 192.168.1.2

Network Plan:

The prototype of the DHCP Server network

IP assigned with DHCP Server

Testing ip config command prompt

DNS Server

Testing Web Hosting

Packet Transfer

Advantages:

- Automated IP Management - DHCP eliminates manual configuration and IP conflicts

- Fast Convergence - EIGRP quickly adapts to network changes and link failures

- Reduced Network Traffic - Triggered updates instead of periodic broadcasts

- Easy Scalability - Simple to add new departments and devices

- Centralized Services - DNS, Web, and FTP servers accessible campus-wide

- Automatic Routing - No manual route configuration needed

- Cost-Effective - Reduces IT staff workload and administrative time

- Fault Tolerance - Automatic rerouting during link failures

- Seamless Communication - All departments interconnected efficiently

Disadvantages

- Single Point of Failure - DHCP server failure prevents new IP assignments

- Vendor Lock-in - EIGRP is Cisco proprietary, limiting hardware choices

- Security Gaps - No firewalls, authentication, or network segmentation implemented

- Complex Configuration - Requires trained personnel and careful setup

Results:

- Successful IP Allocation:

  - All devices received IP addresses automatically from the DHCP server (192.168.1.2).

  - The ipconfig command verified correct IP assignment across departments.

- Effective Routing:

  - The EIGRP protocol successfully established routes between all routers.

  - Fast convergence and minimal packet loss were observed.

- Service Accessibility:

  - The Web server and FTP server were accessible from all departments.

  - The DNS server correctly resolved hostnames.

- Network Performance:

  - Efficient packet delivery with reduced network traffic due to triggered updates in EIGRP.

  - Reliable communication between all connected departments.

References:

- Cisco Networking Academy – CCNA Routing and Switching: Scaling Networks

- Cisco Packet Tracer Documentation

- TutorialsPoint – EIGRP Configuration Guide